ISO/IEC JTC1/SC17		2009-05-27

Cards and personal identification

ISO/IEC JTC1/SC17 N 3681                                                   WG8 N 1548

DOCUMENT TYPE: Notification of Ballot

TITLE: Notification of Ballot - ISO/IEC 14443-1:2008/PDAM 1 - Identification cards - Proximity cards - Part 1: Physical characteristics – Amd 1 - Additional PICC classes.

BACKWARD POINTER:

SOURCE:   SECRETARIAT ISO/IEC JTC1/SC17

STATUS: This ballot has been posted to the ISO Electronic balloting application and is available under the Balloting Portal, Committee Internal Balloting.

(This document is circulated as a Word .doc due to .pdf formatting problems.)

ACTION ID: LB

WORK ITEM: To be advised

DUE DATE: 2009-08-28

DISTRIBUTION:	P and L-Members of ISO/IEC JTC1/SC17

JTC1 Secretariat

ISO/IEC ITTF

MEDIUM:  SERVER

NO. OF PAGES:  8

Secretariat ISO/IEC JTC1/SC17, APACS, Mercury House, Triton Court, 14 Finsbury Square, London EC2A 1LQ, England; Telephone +44 (0)20 7711 6255; Fax: +44 (0)20 7711 6299; e-mail: chris.starr@apacs.org.uk

ISO/IEC JTC 1/SC 17/WG 8  N 1548

Date:   2009-04-22

ISO/IEC 14443-1:2008/PDAM 1

ISO/IEC JJTC 1/SC 17/WG 8

Secretariat:   DIN

Identification cards — Proximity cards — Part 1: Physical characteristics

AMENDMENT 1
Additional PICC classes

Cartes d'identification — Cartes de proximité

AMENDEMENT 1

Classes de PICC additionelles

Warning

This document is not an ISO International Standard. It is distributed for review and comment. It is subject to change without notice and may not be referred to as an International Standard.

Recipients of this draft are invited to submit, with their comments, notification of any relevant patent rights of which they are aware and to provide supporting documentation.

Copyright notice

This ISO document is a working draft or committee draft and is copyright-protected by ISO. While the reproduction of working drafts or committee drafts in any form for use by participants in the ISO standards development process is permitted without prior permission from ISO, neither this document nor any extract from it may be reproduced, stored or transmitted in any form for any other purpose without prior written permission from ISO.

Requests for permission to reproduce this document for the purpose of selling it should be addressed as shown below or to ISO's member body in the country of the requester:

[Indicate the full address, telephone number, fax number, telex number, and electronic mail address, as appropriate, of the Copyright Manger of the ISO member body responsible for the secretariat of the TC or SC within the framework of which the working document has been prepared.]

Reproduction for sales purposes may be subject to royalty payments or a licensing agreement.

Violators may be prosecuted.

Foreword

ISO (the International Organization for Standardization) and IEC (the International Electrotechnical Commission) form the specialized system for worldwide standardization. National bodies that are members of ISO or IEC participate in the development of International Standards through technical committees established by the respective organization to deal with particular fields of technical activity. ISO and IEC technical committees collaborate in fields of mutual interest. Other international organizations, governmental and non-governmental, in liaison with ISO and IEC, also take part in the work. In the field of information technology, ISO and IEC have established a joint technical committee, ISO/IEC JTC 1.

International Standards are drafted in accordance with the rules given in the ISO/IEC Directives, Part 2.

The main task of the joint technical committee is to prepare International Standards. Draft International Standards adopted by the joint technical committee are circulated to national bodies for voting. Publication as an International Standard requires approval by at least 75 % of the national bodies casting a vote.

Attention is drawn to the possibility that some of the elements of this document may be the subject of patent rights. ISO and IEC shall not be held responsible for identifying any or all such patent rights.

Amendment 1 to ISO/IEC 14443-1:2008 was prepared by Joint Technical Committee ISO/IEC JTC 1, Information technology, Subcommittee SC 17, Cards and personal identification.

Identification cards — Proximity cards — Part 1: Physical characteristics

AMENDMENT 1:
Additional PICC classes

Page 3 of ISO/IEC 14443-1:2008, Annex A

Add following new sub clauses after A.1.2:

- A.2	"Class 2"

A "Class 2" PICC shall follow the requirements in A.2.1 and A.2.2.

- A.2.1	Antenna Location

The antenna of a "Class 2" PICC shall be located within a zone defined by two rectangles, as shown in Figure A.2:

- external rectangle: 81 mm x 27 mm;

- internal rectangle: 64 mm x  17 mm, centred in the external rectangle, with 3 mm corner radii,

except for the connections to the ends of the antenna coil, with a maximum area of 300 mm2.

Figure A.2 – Location of the antenna of the "Class 2" PICC

- A2.2	Electrical requirement

The "Class 2" PICC shall also pass the "Class 2" PICC maximum loading effect test defined in ISO/IEC 10373-6.

- A.3"Class 3"

A "Class 3" PICC shall follow the requirements in A.3.1 and A.3.2.

- A.3.1 Antenna Location

The antenna of a "Class 3" PICC shall be located within a zone defined by two rectangles, as shown in Figure A.3:

- external rectangle: 50 mm x 37 mm;

- internal rectangle: 35 mm x 20 mm, centred in the external rectangle, with 3 mm corner radii,

except for the connections to the ends of the antenna coil, with a maximum area of 300 mm2.

Figure A.3 – Location of the antenna of the "Class 3" PICC

- A.3.2	Electrical requirement

The "Class 3" PICC shall also pass the "Class 3" PICC maximum loading effect test defined in ISO/IEC 10373-6.

- A.4	"Class 4"

A "Class 4" PICC shall follow the requirements in A.4.1 and A.4.2.

- A.4.1	Antenna Location

The antenna of a "Class 4" PICC shall be located within a zone defined by two rectangles, as shown in Figure A.4:

- external rectangle: 50 mm x 27 mm;

- internal rectangle: 40 mm x  17 mm, centred in the external rectangle, with 3 mm corner radii,

except for the connections to the ends of the antenna coil, with a maximum area of 300 mm2.

Figure A.4 – Location of the antenna of the "Class 4" PICC

- A.4.2	Electrical requirement

The "Class 4" PICC shall also pass the "Class 4" PICC maximum loading effect test defined in ISO/IEC 10373-6.

- A.5	"Class 5"

A "Class 5" PICC shall follow the requirements in A.5.1 and A.5.2.

- A.5.1	Antenna Location

The antenna of a "Class 5" PICC shall be located within a zone defined by a rectangle of dimensions 40,5 mm x 24,5 mm, as shown in Figure A.5, except for the connections to the ends of the antenna coil, with a maximum area of 300 mm2.

Figure A.5 – Location of the antenna of the "Class 5" PICC

- A.5.2	Electrical requirement

The "Class 5" PICC shall also pass the "Class 5" PICC maximum loading effect test defined in ISO/IEC 10373-6.