| JP1 | Whole documents |  | GE, TE | Though SC17/WG8 members see consensus on the parameters for additional PICC classes (see Resolution 17 in Minutes of the 30th meeting of WG8/TF2; WG8 N 1544), some of the parameters (see below) have not been fixed in the test method (ISO/IEC 10373-6:2001/Amd.9). - Reference PICC antenna external dimensions - Reference PICC settings for PCD Hmin test - Reference PICC settings for PCD Hmax test - LMA compensation factor with present sense coils - Sense coils to use for LMA measurement - Coupling compensation factor with relevant sense coils | Prior to the next ballot stage, fix a test method for additional PICC classes (ISO/IEC 10373-6:2001/Amd.9). |  |
| --- | --- | --- | --- | --- | --- | --- |
| JP2 | Whole documents |  | TE | The alternating magnetic field is defined in ISO/IEC 14443-1:2008, 4.4 (see below). We cannot identify whether the values (“10 A/m” and “12A/m”) also apply to PICCs other than “Class 1” after Classes other than “Class 1” are defined in this amendment. This is because those values can vary when they are measured with another Class of Reference PICC. “4.4 Alternating magnetic field The PICC, whichever form the PICC has according to 4.1, shall continue to operate as intended after continuous exposure to a magnetic field of an average level of 10 A/m rms at 13,56 MHz. The averaging time is 30 seconds and the maximum level of the magnetic field is limited to 12 A/m rms.” | Insert the following description after ISO/IEC 14443-1:2008, 4.4. The values “10 A/m” and “12 A/m” shall be measured with “Class 1” calibration coil only, which values shall also apply to PICCs other than “Class 1”. |  |
| JP3 | Whole documents |  | GE | It is not clear whether there are any known patents regarding the dimension of “Class 2 to 5” PICC. Even though ISO/IEC takes no position concerning the evidence, validity and scope of the patents, the patents known at this time should be disclosed by the proposer. | Disclose the patent list regarding this proposal. |  |
| JP4 | A.3 |  | GE, TE | We do not support the idea of mandating the size of antenna embedded in mobile phone in ISO/IEC. “Class 3” PICC should be studied considering the case where the antenna is embedded in mobile phone, not the case where it is stand-alone. ”Class 3” will be of no use unless RF performance instability is minimized. Specifying antenna size only is not sufficient to minimize the RF performance instability. (1) Mobile phones are designed taking into account of electronic parts implementation, which depends on each product and cannot be standardized globally. (2) The magnetic field received by antenna coil embedded in mobile phone is influenced by the electronic circuits around it. The circuit layout depends on each product and the metallic components may weaken the magnetic field induced by the antenna coil. (3) The technical meaning of the dimension of “Class 3” PICC is not clear. We cannot see what area of mobile phone are “Class 3” antenna intended to be implemented in. | Reconsider the purpose and necessity for standardizing “Class 3” in ISO/IEC. Unless the following requirements are satisfied, “Class 3” should be set to proprietary. - Confirm the dimension of “Class 3” takes into account the influence of the metallic components near the antenna coil. If it has been already considered by the proposer(s), disclose the model of weakening the magnetic field induced by the antenna coil of “Class 3” PICC. - See consensus on the recommended area in mobile phone where “Class 3” antenna is implemented. |  |
| JP5 | Annex A |  | GE | “Additional PICC classes” should not be included in Annex A for interoperability and backward compatibility. | Create new Annex C included “Additional PICC classes”, and Annex C should be informative. |  |