| DE 1 | All clauses |  | ge | Current naming convention for classes is confusing as Class 2 and 3 have the same requirements and same coil area (ID1/2). Naming convention shall be changed. | For the whole document change: "Class 2" into "Class 2a" "Class 3" into "Class 2b" "Class 4" into "Class 3" "Class 5" into "Class 4" |  |
| --- | --- | --- | --- | --- | --- | --- |
| DE 2 | A.2.1 | First paragraph and Figure A.2 | te | PICC antenna zone is too small to allow appropriate antenna design for this Class. Internal rectangle dimensions shall be changed. "Class 2" is renamed to "Class 2a" | Replace: "Internal rectangle: 64 mm × 17 mm,...." By: "Internal rectangle: 64 mm × 13 mm,...." Replace Figure A.2 by Annex DE 1 |  |
| DE 3 | A.3.1 | First paragraph and Figure A.3 | te | PICC antenna zone shall be adapted to fit into classification concept. Class 3 (= Class 2b) is ID1/2 "short" type whereas Class 2 (= Class 2a) is ID1/2 "long" type. Circular coil shapes with the same coil area range shall be defined as well. | Replace text of A.3.1 by: "The antenna of a "Class 2b" shall be located within a zone defined by: - External rectangle: 50 mm × 40 mm; - Internal rectangle: 35 mm × 24 mm, centered in the external rectangle, with 3 mm corner radii, or - External circle: 50 mm diameter; - Internal circle: 32 mm diameter, centered in the external circle, as shown in Figure A.3, except for the connection to the ends of the antenna coil, with a maximum area of 300 mm2." Replace Figure A.3 by Annex DE 2 |  |
| DE 4 | A.4.1 | First paragraph and Figure A.4 | te | PICC antenna zone is too small to allow appropriate antenna design for this Class. Internal rectangle dimensions shall be changed and circular coil shapes with the same coil area range shall be defined as well. "Class 4" is renamed to "Class 3" | Replace text of A.4.1 by: "The antenna of a "Class 3" shall be located within a zone defined by: - External rectangle: 50 mm × 27 mm; - Internal rectangle: 35 mm × 13 mm, centered in the external rectangle, with 3 mm corner radii, or - External circle: 41 mm diameter; - Internal circle: 24 mm diameter, centered in the external circle, as shown in Figure A.4, except for the connection to the ends of the antenna coil, with a maximum area of 300 mm2." Replace Figure A.4 by Annex DE 3 |  |
| DE 5 | A.5.1 | First paragraph and Figure A.5 | te | Minimum antenna zone shall be defined for Class 5 and an additional smaller Class be added to improve interoperability. Circular coil shapes with the same coil area range shall be defined as well. "Class 5" is renamed to "Class 4" | Replace text of A.5.1 by: "The antenna of a "Class 4" shall be located within a zone defined by: - External rectangle: 40,5 mm × 24,5 mm; - Internal rectangle: 25 mm × 10 mm, centered in the external rectangle, with 3 mm corner radii, or - External circle: 35 mm diameter; - Internal circle: 18 mm diameter, centered in the external circle, as shown in Figure A.5, except for the connection to the ends of the antenna coil, with a maximum area of 300 mm2." Replace Figure A.5 by Annex DE 4 |  |
| DE 6 | new subclause A.6 |  | te | An additional smaller Class shall be added as "Class 4" has now defined a minimum antenna zone to complete Class concept. Circular coil shapes with the same coil area range shall be defined as well. | Add new Sublause A.6 as defined in Annex DE 5 |  |

Annex DE 1

Annex DE 2

Annex DE 3

Annex DE 4

Annex DE 5

- A.6	"Class 5"

A "Class 5" PICC shall follow the requirements in A.6.1 and A.6.2.

- A.6.1	Antenna Location

The antenna of a "Class 5" PICC shall be located within a zone, defined by a rectangle of dimensions 25 mm × 20 mm or a circle of 25 mm diameter, as shown in Figure A.6, except for the connections to the ends of the antenna coil, with a maximum area of 300 mm2.

Figure A.6 – Location of the antenna of the "Class 5" PICC

- A.6.2	Electrical requirement

The "Class 5" PICC shall also pass the "Class 5" PICC maximum loading effect test defined in ISO/IEC 10373-6.