| JP1 | Whole documents |  | GE, TE | Though SC17/WG8 members see consensus on the parameters for additional PICC classes (see Resolution 17 in Minutes of the 30th meeting of WG8/TF2; WG8 N 1544), some of the parameters (see below) have not been fixed in the test method (ISO/IEC 10373-6:2001/Amd.9). - Reference PICC antenna external dimensions - Reference PICC settings for PCD Hmin test - Reference PICC settings for PCD Hmax test - LMA compensation factor with present sense coils - Sense coils to use for LMA measurement - Coupling compensation factor with relevant sense coils | Prior to the next ballot stage, fix a test method for additional PICC classes (ISO/IEC 10373-6:2001/Amd.9). |  |
| --- | --- | --- | --- | --- | --- | --- |
| JP2 | 6.2 |  | GE, TE | We do not support the idea of mandating the operating field of “Class 3” in ISO/IEC. “Class 3” PICC should be studied considering the case where the antenna is embedded in mobile phone, not the case where it is stand-alone. “Class 3” will be of no use unless RF performance instability from the following description is minimized. Specifying the operating field only is not sufficient to minimize the RF performance instability. (1) Mobile phones are designed taking into account of electronic parts implementation, which depends on each product and cannot be standardized globally. (2) The magnetic field received by antenna coil embedded in mobile phone is influenced by the electronic circuits around it. The circuit layout depends on each product and the metallic components may weaken the magnetic field induced by the antenna coil. | Reconsider the purpose and necessity for standardizing “Class 3” in ISO/IEC. Unless the following requirement is satisfied, operating field for “Class 3” should be set to proprietary - Confirm the operating field of “Class 3” takes into account the influence of the metallic components near the antenna coil. If it has been already considered by the proposer(s), disclose the model of weakening the magnetic field induced by the antenna coil of “Class 3” PICC. |  |
| JP3 |  |  | GE | “Additional PICC classes” should not be included in basic clauses for interoperability and backward compatibility. | Create new Annex B included “Additional PICC classes”, and Annex B should be informative. |  |