| DE 1 | 6.2 |  | te | Classes are renumbered by German comments to CD 14443-1 Amd1 and an additional smaller Class is introduced. The text of this amendment shall be a updated accordingly. | Replace text above Table 1 by: "The PCD shall generate a field strength of at least Hmin and not exceeding Hmax at manufacturer specified positions (operating volume) under unmodulated conditions. PCD requirements measured with Reference PICCs 1, 2a and 2b are mandatory for all PCDs. PCD requirements measured with Reference PICC 3 are only mandatory for PCDs supporting optional operation with Class 3 PICCs. PCD requirements measured with Reference PICC 4 are only mandatory for PCDs supporting optional operation with Class 4 PICCs. PCD requirements measured with Reference PICC 5 are only mandatory for PCDs supporting optional operation with Class 5 PICCs. |  |
| --- | --- | --- | --- | --- | --- | --- |
| DE 2 | 6.2 | Table 1, 2 | te | Table 1 and 2 shall be extended to include additional smaller Class requirements. Classes are renumbered. Operating field strength range (Hmax - Hmin) shall be constant. | Replace Table 1 by Annex DE1 and Table 2 by Annex DE2 |  |

Annex DE 1

Table 1 — PCD field strength

|  | PCD |  |
| --- | --- | --- |
|  | Hmin | Hmax |
| Measured with Reference PICC 1 | 1,5 A/m (rms) | 7,5 A/m (rms) |
| Measured with Reference PICC 2a | 1,5 A/m (rms) | 7,5 A/m (rms) |
| Measured with Reference PICC 2b | 1,5 A/m (rms) | 7,5 A/m (rms) |
| Measured with Reference PICC 3 (optional) | 2,0 A/m (rms) | 8 A/m (rms) |
| Measured with Reference PICC 4 (optional) | 2,5 A/m (rms) | 8,5 A/m (rms) |
| Measured with Reference PICC 5 (optional) | 4,5 A/m (rms) | 10,5 A/m (rms) |

Annex DE 2

Table 2 — PICC operating field strength

|  | PICC |  |
| --- | --- | --- |
|  | Hmin | Hmax |
| "Class 1" | 1,5 A/m (rms) | 7,5 A/m (rms) |
| "Class 2a" | 1,5 A/m (rms) | 7,5 A/m (rms) |
| "Class 2b" | 1,5 A/m (rms) | 7,5 A/m (rms) |
| "Class 3" | 2,0 A/m (rms) | 8 A/m (rms) |
| "Class 4" | 2,5 A/m (rms) | 8,5 A/m (rms) |
| "Class 5" | 4,5 A/m (rms) | 10,5 A/m (rms) |