| DE 1 | Subclause 8.1 |  | te | The time period for the PICC to execute a command is too low and shall be increased. | Replace: "This low EMD time tE, PICC has a value of F – 800/fc with a maximum value of 1288/fc where F equals FDT PCD to PICC for type A and TR0 for type B. The minimum value is 0/fc for TR0 ≤ 800/fc." By: "This low EMD time tE has a value of F – 1024/fc with a maximum value of 1288/fc where F equals FDT PCD to PICC for type A and TR0 for type B. The minimum value is 0/fc for TR0 ≤ 1024/fc." Replace: "This low EMD time tE, PCD has a value of F – 868/fc with a maximum value of 1220/fc where F equals FDT PCD to PICC for type A and TR0 for type B. The minimum value is 0/fc for TR0 ≤ 868/fc." By: "This low EMD time tE, PCD has a value of F – 1044/fc with a maximum value of 1220/fc where F equals FDT PCD to PICC for type A and TR0 for type B. The minimum value is 0/fc for TR0 ≤ 1044/fc." |  |
| --- | --- | --- | --- | --- | --- | --- |
| DE 2 | Subclause 8.2 | NOTE | te | PCD Values in NOTE shall be same as in 8.1. | Replace: " NOTE To avoid unnecessary reception of EMD, PCDs need not be ready to start receiving PICC frames less than 868/fc after the end of their command frames (unless for type B when minimum TR0 has been reduced)." By: " NOTE To avoid unnecessary reception of EMD, PCDs need not be ready to start receiving PICC frames less than 1044/fc after the end of their command frames (unless for type B when minimum TR0 has been reduced)." |  |
| DE 3 | subclause 8.2 | 3rd paragraph and following text | ge | This paragraph belongs to ISO/IEC 14443-4. Starting from: Because EMD handling requires the PCD to distinct between likely EMD and transmission errors the PCD should implement a more precise handling of errors in the following scenarios. | Remove 3rd paragraph and start the procedure for a new Amendment to ISO/IEC 14443-4. See separate contribution from Germany. |  |