| FR1 | Contents |  |  |  | ed |  | Table of content is not needed for such a small amendment |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FR2 | Page 5, clause 4 |  | 1st line |  | ed |  | typo |  | Replace "symbol" by "symbols" |  |  |  |
| FR3 | Definition |  |  |  | te |  |  |  | Remove all references to VE (VE,PICC and VE,PCD) |  |  |  |
| FR4 |  | 7.7.3 |  | AFI family 'E' and last paragraph |  | te |  | Sub family 'EY' is RFU if Y is neither '1' neither '2'. Then a PICC should not answer to AFI 'EY' if Y different to '1' and to '2'. All sub families 'E' should be allowed like other sub families. |  | Replace "'0', Y = 1, Y = 2, Other Y values are RFU" With '0', Y |  |  |
| FR5 | 8.1 |  | 1st paragraph |  | te |  |  |  | Replace "the EMD limit VE,PICC as defined in ISO/IEC 14443-2/Amendment 3" With "the EMD limit as defined in ISO/IEC 14443-2/Amendment 3" |  |  |  |
| FR6 | 8.2 |  | 3rd paragraph |  | te |  |  |  | Replace "above VE,PCD" With "the EMD limit as defined in ISO/IEC 14443-2/Amendment 3" |  |  |  |
| FR7 | 8.1 |  | 2nd paragraph |  | ed |  | Readability |  | Split this paragraph after "800/fc." so that the sentence "The low EMD time tE, PCD is the time period to allow the PCD to recover from electromagnetic disturbances." is an independent paragraph |  |  |  |
| FR8 | 8.1 |  |  |  | te |  | The low EMD time is slightly too short for some existing readers to recover. In addition, the maximum value of PICC low EMD time (used when FDT/TR0 is long enough) has been defined to a higher value (1408/fc) by EMVCo in its latest specification V2.0.1. |  | Change the maximum required value of PICC low EMD time to 1408/fc (103,8 µs). And the maximum required value of PCD low EMD time to 1340/fc (98,8µs) to keep the same margin. |  |  |  |