| JP1 | Whole documents |  | GE | As the revision of ISO/IEC 14443-2 is not published at 2010-02-10, this document should be an amendment to ISO/IEC 14443-2:2001 instead of ISO/IEC 14443-2:2010. | Use the term “ISO/IEC 14443-2:2001/Amd.4” until the revision of ISO/IEC 14443-2 is published. |  |
| --- | --- | --- | --- | --- | --- | --- |
| JP2 | Whole documents | Title | ED | The title of this document is wrong. | Replace “Identification cards — Proximity cards — Part 2: Radio frequency power and signal interface” by “Identification cards — Contactless integrated circuit cards - Proximity cards — Part 2: Radio frequency power and signal interface”. |  |
| JP3 | 6 | 1st Paragraph | ed | For correction. | Remember to remove of “FCD” of “Page 4 of FCD ISO/IEC 14443-2, Clause 6”, after the revision of ISO/IEC 14443-2 is published. |  |
| JP4 | 6.2 |  | te | Even though optionally support, PCD is allowed to generate 7.5A/m and over. But legacy PICCs would be damaged by such a strong field. There is a possibility that such a strong field could damage conventional infrastructure. Counter measures to protect PICCs from breaking down are needed. For any measures, warnings to maintain the safety of products should at lease be noted. | The following two cases should be significantly noted for example: 1) Permutation between Class1 PICC and PCD generating strong field for class4, 5 and 6. 2) Relatively long time expose PICCs to PCD generating strong field for class4, 5 and 6. |  |
| JP5 | 6.2 | 2nd Paragraph | ed | Optionally support PICCs are PICC Class at least one of 4, 5 and 6. | Replace “PICCs of classes 4 and /or 5” with “PICCs of classes at least one of 4, 5 and 6”. |  |
| JP6 | 6.2 | 2nd Paragraph | ed | For consistency regarding the description for “Class 1,2,3,4,5 and/or 6”. | Replace “classes 1, 2 and 3” and “classes 4 and/or 5” by “”Class 1”, “Class 2” and “Class 3”” and “”Class 4” ,“Class 5” and/or “Class 6””, respectively. |  |
| JP7 | 6.2 | 4th Paragraph | ed | For consistency regarding the description for “Class 1,2,3,4,5 and/or 6”. | Replace “Class 4 PICCs” by “”Class 4” PICCs”. |  |
| JP8 | 6.2 | 5th Paragraph | ed | For consistency regarding the description for “Class 1,2,3,4,5 and/or 6”. | Replace “Class 5 PICCs” by “”Class 5” PICCs”. |  |
| JP9 | 6.2 | 6th Paragraph | ed | For consistency regarding the description for “Class 1,2,3,4,5 and/or 6”. | Replace “Class 6 PICCs” by “”Class 6” PICCs”. |  |
| JP10 | 6.2 | The paragraph just before Table 2 | ed | For consistency regarding the description for “Class 1,2,3,4,5 and/or 6”. | Replace “Class 1” by “”Class 1””. |  |