| US | 3.2 | 3.2 | T | CCF is a new term. What is the unit value of CCF? | Please add a unit value indicator and an explanation of the term CCF. |  |
| --- | --- | --- | --- | --- | --- | --- |
| US | Annex (new) | Annnex (new) | T | Calibration coils are useful tools for evaluating PCDs in both laboratory and field environments because of their simplicity and ease of use. Using the Class 1 sized calibration coil currently defined in ISO/IEC10373-6 for PCD measurements regarding Class 2, 3, 4, or 5 PICCs on real PCDs is inappropriate. A calibration coil measures the average magnetic flux within the area of the coil; if the calibration coil does not have approximately the same outer dimensions as the PICC antenna, then the measurement may not accurately reflect the magnetic field available to the PICC. Note that the US is willing to draft the proposed new informative annex if desired by WG8 and the project editor. | Please add a new informative Annex for the test setup. Provide a procedure for measurement of unloaded magnetic field strength on PCDs using calibration coils corresponding to each class of PICC. State that these procedures are not intended as substitutes for the PCD conformance tests in section 7.1. However this procedure may be used to evaluate field strength and uniformity in environments where application of the section 7.1 tests is not practical. State that real PCDs generate fields that are not as uniform as the ISO10373 test PCD field and that only calibration coils with dimensions similar to the PICC antenna can accurately measure the magnetic field the PICC sees when it is on a real PCD. |  |
| US | 5.4.3 | 5.4.3 | E | 5.4.3. (page 4); the reference should be to steps f, g, I and not to 6, 7, 9 | Please change the reference to steps to: f, g and l. |  |
| US | 7.1.1.2 | 7.1.1.2 | E | 7.1.1.2. (page 5); add corresponding H max corrections for V load | Please add the corresponding H max corrections for V load. |  |
| US | 5.4.2 | 5.4.2 | E | 5.4.2 (reference to page 10 of FCD 10373-6:2009) shall be changed to read: “shall have minimal, if any, effect in the RF measurements.” | See Comment Field |  |
| US |  |  | T | US submitted comments (file:Nov_04_09-straw_poll.doc); file WG8N1627 shows the disposition of comments. The US decided to resubmit the following comments: US 1, US 2, US 3, and US 4 with examples of applications |  |  |