| DE 1 | 7.1.7.2 | step c) | te | Only adjustable load shall be used for this test same as for load modulation test in FDIS 10373-6. | Delete: " Switch the jumper J1 to 'b' and adjust R2 to obtain a VLOAD of 6V (DC) measured at CON3." Change "Alternatively,...." to: " Switch jumper J1 to position 'c' and adjust the DC voltage at CON2 to obtain a VLOAD of 6V (DC) at CON3. The operating field condition shall be verified by monitoring the voltage on the calibration coil." |  |
| --- | --- | --- | --- | --- | --- | --- |
| DE 2 | 7.1.7.2 | step f) | te | Tests shall be performed with minimum FDT/TR0 and maximum FDT/TR0 to cover complete FDT/TR0 range. | Change step f) first paragraph to: Send in sequence, as illustrated in Figure Amd.9.4 using the tE, PCD associated with minimum FDT/TR0. Add new step h): h) Repeat step a) to g) with maximum FDT/TR0. Previous step h) becomes new step i): i) Repeat step a) to h) 10 times. |  |
| DE 3 | 7.1.7.2 |  | te | The PICC Answer in Type B shall use min TR1 according to ISO/IEC 14443-2 9.2.5 to apply a worst case scenario. A NOTE shall be added. | Add a NOTE after Figure AMD.9.4: NOTE: The PICC answer shall use minimum TR1 as defined in ISO/IEC 14443-2:2010 9.2.5. |  |
| DE 4 | 7.1.7.2 | Figure AMD.9.5 | te | Figure represents only bit rate fc/128. A corresponding definition and figure for bit rates higher than fc/128 is missing. | Add second figure for Type A for bit rates higher than fc/128 or remove "no subbcarrier" |  |
| DE 5 | 7.1.7.2 | Figure AMD.9.6 | te | SOF starts with no subcarrier. This is not possible in type B. A response always starts with subcarrier for a time TR1. | Remove "no subcarrier" |  |