“Class 1” definition

Here is the definition of Class 1 requirements as agreed during the 21st TF2 meeting, updated as agreed during the 23rd TF2 and 33rd WG8 meetings:

PICC additional requirements

Antenna zone (to be included in next revision of ISO/IEC 14443-1)

The PICC antenna shall be entirely located within a zone defined by two rectangles:

- external rectangle: 81 mm x 49 mm, centred in ID1

- internal rectangle : 64 mm x 34 mm, centred in ID1, with 3 mm radius

Maximum loading effect (included in ISO/IEC 10373-6/PDAM4, 7.4)

The PICC loading effect at 1,5 A/m shall be measured using the test PCD assembly. It shall not exceed the loading effect of the reference PICC (Annex D) calibrated for 6 volts DC at 1,5 A/m.

PCD additional requirement

Minimum field strength test (included in ISO/IEC 10373-6/PDAM4, 8.5)

Hmin test defined in clause 8.1 of ISO/IEC 10373-6:2001 shall be repeated using VDC = 6V (dc) instead of 3V (dc) on the reference PICC for (Annex D), both for step 3 and step 4.